Combinational Hardware Trojan Insertion in an Obfuscated 4-Bit Ripple Carry Adder

# Objective

The objective of this experiment is to investigate the insertion and behavior of a combinational hardware Trojan within an obfuscated 4-bit ripple carry adder implemented in VHDL. Students will analyze a provided combinationally obfuscated ripple carry adder, design and implement a trigger condition and payload, simulate the Trojan-infected design, and analyze the resulting behavior through waveform inspection.

Upon completion of this experiment, students should be able to:

• Explain the purpose of hardware obfuscation and hardware Trojans

• Describe the operation of a 4-bit ripple carry adder

• Explain XOR/XNOR-based and multiplexer-based combinational obfuscation

• Distinguish between Trojan trigger conditions and Trojan payloads

• Implement a combinational trigger condition in VHDL

• Implement a Trojan payload that modifies a locking key

• Verify Trojan activation through simulation

• Analyze the effect of a hardware Trojan on circuit functionality

• Detect Trojan activation using simulation waveforms

# Case Study

Modern integrated circuits are frequently designed by one organization and fabricated by another. During design, verification, fabrication, testing, and deployment, hardware intellectual property is exposed to numerous security threats.

Common threats include:

• Reverse engineering

• Intellectual property theft

• Unauthorized overproduction

• Malicious circuit modification

One particularly dangerous threat is the insertion of a hardware Trojan. A hardware Trojan is a malicious modification to a circuit that remains dormant until activated by a specific trigger condition. Once activated, the Trojan executes a payload that alters the intended functionality of the design.

In this experiment, students are provided with an already obfuscated 4-bit ripple carry adder. Rather than constructing the obfuscation circuitry themselves, students will modify the design by inserting a combinational hardware Trojan. The Trojan will activate when a specific input combination occurs and will modify the locking key used by the obfuscation circuitry. The resulting corruption of the key will cause the ripple carry adder to produce an incorrect output despite the correct key being supplied.

# Theory Background

## 1. Ripple Carry Adder

A ripple carry adder is a digital circuit used to perform binary addition. It is constructed by connecting multiple full adders in series.

Each full adder accepts:

• Input A

• Input B

• Carry-In (Cin)

and produces:

• Sum

• Carry-Out (Cout)

The carry output from one stage becomes the carry input of the next stage. Because the carry propagates through each stage, the structure is referred to as a ripple carry adder.

In this laboratory, the ripple carry adder is constructed using four full adder components connected in series.

Figure 1. Structure of a 4-bit Ripple Carry Adder

## 2. Hardware Obfuscation

Hardware obfuscation protects digital intellectual property by modifying a design such that it only functions correctly when a valid unlocking condition is satisfied.

The primary goals of hardware obfuscation include:

• Prevention of reverse engineering

• Protection against IP piracy

• Prevention of unauthorized manufacturing

• Increased resistance against malicious modification

## 2.1 Combinational Logic Locking

Combinational logic locking modifies a circuit by inserting key-controlled logic into the design.

The circuit operates correctly only when the proper key is applied. An incorrect key causes the circuit to generate incorrect outputs.

Because the locking mechanism depends only on current inputs and key values, no memory of previous states or inputs is required.

## 2.2 XOR/XNOR-Based Obfuscation

One common method of combinational obfuscation inserts XOR and XNOR gates into critical signal paths.

For XOR:

A XOR 0 = A

For XNOR:

A XNOR 1 = A

Therefore, the proper key value allows the original signal to propagate unchanged.

Incorrect key values invert the signal and corrupt circuit functionality.

In the provided design, XOR and XNOR gates are inserted between ripple carry stages.

Figure 2. XOR/XNOR Obfuscated Ripple Carry Adder

## 2.3 Multiplexer-Based Obfuscation

Multiplexer-based obfuscation uses key-controlled multiplexers to select between the original carry signal and an alternate signal.

When the correct key is applied, the original carry propagates normally.

When an incorrect key is applied, an alternate signal is selected, corrupting carry propagation and producing incorrect outputs.

Figure 3. Multiplexer Obfuscated Ripple Carry Adder

## 3. Hardware Trojans

A hardware Trojan is a malicious modification inserted into a digital circuit.

Unlike combinational logic locking, which is intended to protect intellectual property, hardware Trojans are intended to alter circuit behavior without the designer’s knowledge.

A Trojan generally consists of two components:

- Trigger Condition

- Payload

The Trojan remains inactive until its trigger condition is satisfied.

Once activated, the payload modifies the circuit behavior.

## 3.1 Trigger Conditions

A trigger condition determines when a Trojan activates.

Triggers may be based on:

• Input combinations

• Internal signals

• Counters

• State machines

In this experiment, the trigger will be based on a unique combination of ripple carry adder inputs.

To simplify waveform analysis, students should select an easily recognizable input combination.

Example:

A = 0011 (3)

B = 0101 (5)

Students may choose another unique combination with instructor approval.

## 3.2 Trojan Payloads

The payload determines what action the Trojan performs after activation.

Possible payloads include:

• Inverting a key bit

• Forcing a key bit to logic 0

• Forcing a key bit to logic 1

• Swapping key bits

In this laboratory, the payload must modify one or more key bits before they are applied to the existing obfuscation circuitry.

Example payload:

Invert Key(0) when the trigger condition is active.

Because the obfuscation circuitry depends on the key, modifying the key causes the ripple carry adder to produce incorrect outputs.

# Provided Files

Students will be provided with:

• Full Adder source code

File: FullAdder.vhdl

• Obfuscated Ripple Carry Adder source code

File: 4BitAdder_Key.vhdl

• Testbench template

File: tb_4BitAdder_Key.vhdl

The provided design already contains combinational obfuscation.

Students are not required to modify the obfuscation circuitry.

Students are only responsible for implementing the Trojan trigger and payload.

# Experiment Setup

## Software Requirements

- Vivado Design Suite

- Provided VHDL Source Files

- Simulation Waveform Viewer

## Knowledge Requirements

Students should be familiar with:

- VHDL

- Full Adders

- Ripple Carry Adders

- Component Instantiation

- Behavioral Simulation

- Hardware Obfuscation

# Instructions and Questions

# Part I – Analysis of the Provided Obfuscated Design

Students shall analyze the provided combinationally obfuscated ripple carry adder.

## Tasks

- Examine the provided VHDL source code.

- Identify the key input signals.

- Determine how carry signals are obfuscated.

- Simulate the design using the provided key.

- Verify correct addition behavior.

## Deliverables

- Annotated VHDL source code

- Obfuscated ripple carry adder schematic

- Simulation waveform

- Brief explanation of the obfuscation mechanism

## Questions

- What is the purpose of the key input?

- Why are carry signals effective locations for obfuscation?

- How does XOR/XNOR locking affect carry propagation?

- How does multiplexer locking affect carry propagation?

# Part II – Trojan Trigger Design

Students shall design a combinational trigger condition.

## Requirements

- Select a unique combination of A and B.

- The trigger shall activate only for that combination.

- The trigger shall be implemented as combinational logic.

- The trigger shall be represented by a signal named trigger.

## Deliverables

- Trigger condition VHDL code

- Description of the selected trigger combination

- Simulation demonstrating trigger activation

## Questions

- Why is the selected trigger combination unlikely to occur accidentally?

- How could a trigger be made more difficult to detect?

# Part III – Trojan Payload Implementation

Students shall implement a Trojan payload.

## Requirements

- Create a modified key signal.

- The payload shall execute only when trigger = ‘1’.

- The payload must modify at least one key bit.

- The existing obfuscation circuitry shall remain unchanged.

## Deliverables

- Modified VHDL source code

- Description of the payload

- Updated schematic

## Questions

- Which key bit was modified?

- How does the payload affect the obfuscation circuitry?

- Why does the output become corrupted?

# Part IV – Trojan Verification

Students shall verify Trojan operation through simulation.

## Test Case A – Trigger Inactive

Apply input values that do not activate the trigger.

Expected Result:

The ripple carry adder shall operate normally.

## Test Case B – Trigger Active

Apply the selected trigger input combination.

Expected Result:

The payload shall modify the key and corrupt the output sum.

## Waveform Requirements

Capture waveforms showing only:

• A

• B

• S

• Co

Change the waveform radix to Unsigned Integer before taking screenshots.

Clearly indicate:

• Normal operation

• Trojan activation

• Corrupted output

# Part V – Trojan Detection and Analysis

Students shall analyze simulation results and identify Trojan behavior.

## Tasks

- Identify when the Trojan activates.

- Determine which key bit was modified.

- Explain how the modified key propagates through the obfuscation circuitry.

- Compare outputs before and after Trojan activation.

- Explain why the Trojan remains hidden for all other input combinations.

## Deliverables

- Annotated waveforms

- Trojan analysis report

- Explanation of trigger and payload interaction

# Lab Report Guidelines

## Part I

Include:

- Annotated source code

- Obfuscated RCA schematic

- Simulation waveform

- Obfuscation explanation

## Part II

Include:

- Trigger implementation

- Trigger description

- Trigger verification waveform

## Part III

Include:

- Payload implementation

- Modified source code

- Updated schematic

## Part IV

Include:

- Testbench source code

- Normal operation waveform

- Trojan activation waveform

- Discussion of simulation results

## Part V

Include:

- Trojan detection analysis

- Trigger explanation

- Payload explanation

- Security discussion

# Discussion Questions

- Explain the difference between hardware obfuscation and hardware Trojans.

- What is the purpose of a trigger condition?

- What is the purpose of a Trojan payload?

- Why does modifying the key produce an incorrect output?

- How could a Trojan be made more difficult to detect?

- Why are rare trigger conditions effective?

- Compare combinational hardware Trojans and sequential hardware Trojans.

- How does the existing obfuscation circuitry amplify the effect of the Trojan payload?